SHREYA PANCHAL

N+ Network security
    Theme: Startup Office IT Setup

Section A: Scenario-Based Concept Application

1. Explain the selection process for CPU, RAM, and storage specifications to ensure hardware compatibility and prevent performance bottlenecks for a startup team performing concurrent development and video conferencing tasks.

Answer: To avoid performance problems, the hardware should match the team's workload.

- CPU: Choose a multi-core processor (Intel Core i5/i7 or AMD Ryzen 5/7) so developers can run multiple applications while attending video meetings.

- RAM: Install at least 16 GB RAM. For heavy development work, 32 GB is recommended.

- Storage: Use an SSD (512 GB or 1 TB) because it provides faster boot time and quick application loading than an HDD.

- Compatibility: Ensure the CPU is compatible with the motherboard socket, RAM type (DDR4/DDR5), and storage interface (SATA or NVMe).

Conclusion: Selecting compatible CPU, RAM, and SSD improves performance and prevents bottlenecks.

2. Explain how system resource allocation and hardware bottlenecks contribute to performance degradation after the deployment of resource-intensive background applications like antivirus software.

Answer: When antivirus software runs in the background, it continuously uses CPU, RAM, and disk resources.

- High CPU usage slows down applications.

- Low available RAM causes excessive paging to disk.

- Heavy disk activity reduces file access speed.

- Users may experience slow boot, lagging applications, and delayed response.

Conclusion: Proper resource allocation and sufficient hardware prevent performance degradation.

3. Explain the systematic troubleshooting process for resolving peripheral hardware failures caused by driver incompatibility or operating system conflicts when a device functions inconsistently across different workstations.
Answer: A systematic troubleshooting process includes:

- Check cable and power connections.

- Test the device on another computer.

- Open Device Manager and check for driver errors.

- Update or reinstall the correct driver.

- Install operating system updates.

- Restart the computer.

- Test the peripheral again.

If the device works on one system but not another, the issue is usually related to drivers or operating system conflicts.

4. Explain the role of Uninterruptible Power Supply (UPS) systems and surge protection in preventing file system corruption and ensuring data integrity during unexpected power fluctuations.

Answer: A UPS (Uninterruptible Power Supply) provides temporary backup power during power failures.

Benefits:

- Prevents sudden shutdowns.

- Protects important files from corruption.

- Gives users time to save work.

- Maintains data integrity.

A surge protector protects computer hardware from voltage spikes that may damage components.

Conclusion: UPS and surge protection increase system reliability and protect valuable data.

Section B: Practical Application

Task 1 : physically assemble a workstation and document the component installation sequence.

Answer: Component Installation Sequence

- Open CPU cabinet.

- Install SMPS (Power Supply).

- Install Motherboard.

- Install CPU.

- Apply Thermal Paste.

- Install CPU Fan/Heatsink.

- Install RAM.

- Install SSD/HDD.

- Connect SATA Cable (SSD/HDD → Motherboard).

- Connect Power Cables (24-pin & CPU 8-pin).

- Connect Front Panel Cables (Power SW, Reset SW, HDD LED).

- Install Graphics Card (if required).

- Close cabinet.

- Connect Monitor, Keyboard, Mouse.

- Power ON and check POST.

Expected Result:

- Computer starts successfully.

- BIOS opens without errors.

Task 2 : Troubleshoot and resolve a RAM mismatch error within the BIOS/UEFI settings.

Answer: Troubleshoot RAM Mismatch Error in BIOS/UEFI

Problem: RAM not detected or memory mismatch.

Steps:

- Restart PC.

- Press Delete or F2 to enter BIOS.

- Check installed RAM size.

- If incorrect:

  - Turn OFF PC.

  - Remove RAM.

  - Clean RAM contacts.

  - Reinsert RAM properly.

- If two RAM sticks:

  - Install in recommended slots (A2 & B2).

- Enable XMP/DOCP (if supported).

- Save using F10.

- Restart.

Expected Result:

- BIOS shows correct RAM.

- No memory error during POST.

Task 3 : Build a LAN topology in Packet Tracer for 5 systems and 1 printer using a Class C subnet.

Answer: Build LAN Topology in Cisco Packet Tracer

Devices:

- 5 PCs

- 1 Printer

- 1 Switch (2960)

Connections:

Use Copper Straight Through Cable

- PC0 → Switch

- PC1 → Switch

- PC2 → Switch

- PC3 → Switch

- PC4 → Switch

- Printer → Switch

IP Addressing (Class C):

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |
| PC2 | 192.168.1.3 | 255.255.255.0 |
| PC3 | 192.168.1.4 | 255.255.255.0 |
| PC4 | 192.168.1.5 | 255.255.255.0 |
| Printer | 192.168.1.10 | 255.255.255.0 |

(Default Gateway is not required unless connecting to another network.)
Verify Connectivity: From PC0

ping 192.168.1.2
ping 192.168.1.3
ping 192.168.1.4
ping 192.168.1.5
ping 192.168.1.10

Expected Result:

Reply from 192.168.1.x

All pings should be successful.

Task 4: Perform a clean OS installation with manual disk partitioning and startup optimization.

Answer: Clean OS Installation (Windows/Linux)

Step 1: Create bootable USB.

Step 2: Boot from USB.

Step 3: Select Custom Installation

Step 4: Manual Partition

Example:

| Partition | Size |
| --- | --- |
| C Drive | 80 GB |
| D Drive | Remaining |

Step 5: Install OS.

Step 6 Startup Optimization

Windows

- Disable unnecessary Startup Apps.

- Install Drivers.

- Run Windows Update.

Success Criteria: System passes POST, all IPs are reachable via Ping, and the OS boots without errors.
Answer: Success Criteria

POST completes successfully (no beep/error).
BIOS detects all hardware (CPU, RAM, Storage).
All devices in Packet Tracer can communicate using ping.
OS installs successfully.
System boots normally without errors.

Section C: Integrated Case Study

1. Identify the three hardware components most likely to cause random reboots under load.
Answer: The three hardware components most likely to cause random reboots under heavy load are:

- RAM (Memory)

  - Faulty or incompatible RAM can cause system crashes and random restarts, especially during multitasking or heavy applications.

- Power Supply Unit (PSU/SMPS)

  - If the PSU cannot provide stable power under high load, the computer may suddenly reboot or shut down.

- CPU (Processor) / CPU Cooling System

  - An overheating CPU (due to a faulty fan, poor thermal paste, or inadequate cooling) can trigger automatic restarts to protect the hardware.

2. Explain why the systems pass POST in the morning but fail in the afternoon. 
Answer: Key Reason: POST uses minimal hardware load 
 Explanation:

During POST (Power-On Self-Test):

CPU runs at low load

GPU is barely active

RAM and storage are lightly accessed

Therefore, no overheating or power stress occurs   Afternoon Failure Cause:

Heavy rendering workloads increase:

CPU usage to 90–100%

GPU utilization to maximum

Power draw spikes

Poor ventilation in desk cabinets traps heat  
Result:

Temperature rises gradually

Thermal throttling occurs

Eventually system triggers automatic shutdown or restart

3. How would you use a stress-test tool to confirm a thermal or power issue? 
    Answer: Recommended Tools: Prime95 (CPU stress test)  
                       FurMark (GPU stress test)  
                            AIDA64 / HWMonitor (temperature monitoring)  
                       OCCT (CPU + PSU stability test)

Procedure:

Install monitoring tool (HWMonitor or AIDA64)  Start temperature monitoring:  
CPU temperature  
GPU temperature Voltage readings 
Run stress test:  
CPU: Prime95 (10–20 minutes)
GPU: FurMark (burn-in test)  
Observe system behavior:  
Sudden restart → PSU issue likely  
High temperature (90°C+) → cooling issue  
Thermal throttling → CPU/GPU overheating

Expected Indicators:

| Symptom | Cause |
| --- | --- |
| Instant reboot under load | PSU failure |
| Gradual slowdown + shutdown | Thermal issue |
| GPU artifacts + crash | GPU overheating |

- Propose three specific steps to remediate the frequent system restarts. 
 Answer:1. Improve Cooling and Airflow

Remove systems from enclosed desk cabinets  
Clean dust from fans and heatsinks  
Add additional case fans or upgrade CPU cooler

2. Thermal Paste Reapplication

Remove CPU cooler  
Clean old thermal paste  
Apply fresh high-quality thermal compound  
Reinstall cooler properly

3. Power and Load Optimization

Replace low-quality PSU if needed  
Ensure PSU wattage matches system requirements  
Balance workload distribution (CPU/GPU optimization)  
Avoid full sustained rendering loads in poorly cooled environment.

- Provide a brief technical justification for why a PSU upgrade might (or might not) be necessary. 
 Answer: When PSU Upgrade IS Necessary:

System restarts occur under high GPU/CPU load  
Voltage instability detected in stress tests  
PSU wattage is close to system maximum requirement  PSU is low quality or aging  
Reason:  Rendering workloads cause sudden power spikes. If PSU cannot maintain stable voltage rails (12V, 5V), system protection circuits trigger a restart.

When PSU Upgrade is NOT Necessary:

Temperatures are the real cause (not power)  
PSU wattage is sufficient and stable Stress test shows thermal shutdown only  
Reason: In this case, the issue is thermal throttling, not power delivery.